Solution to Coastal Disasters Conference

Track:   Coastal Storms

Area of Expertise: coastal flooding

Large Scale Coastal Storm Hazard Mapping

C.C. Giarrusso†, E.Pugliese Carratelli* & G. Spulsi‡

†	Department of Environmental and Civil Engineering, University of Catania (ITALY)

### currently at National University of Singapore, TMSI, tmsgcc@nus.edu.sg.

*	E.Pugliese Carratelli c/o Department of Civil Engineering, University of Salerno - EPC@UNISA.IT Fax +39-89-964045

* Corresponding author

‡	EtaBeta, Naples ITALY

Keywords: Coastal flooding, hazard identification, mapping and analysis

The paper will report on the experience gained by the Authors during the compilation of Civil Protection Contingency Plans for Italian Local and Watershed Authorities, as well as on the related research activities.

.

The main task carried out in connection with such work was the drawing up of hazard maps for different time horizons (e.g. 3 and 10 years) and different damage levels;  the problem associated with both coastal erosion forecasting (which affects long term risks) and wave run-up (which determines short term risks and hazard levels) will be quickly reviewed with a strong emphasis on the operational application.  Thus no discussion will be provided about the relative efficiency of long-shore erosion models – about which a wealth of literature already exists – while some information will be provided about the use of laser altimetry, ortophoto imagery and conventional mapping techniques in providing data on the coastline position.

A special emphasis will be given on the techniques used to evaluate run up risk on low coasts, by comparing results given by standard formulae with those obtained by making use of 2D shallow water numerical models of waves;  details on model calibration techniques will be given and the connection between the model output and the potential damage to coastal infrastructure will be discussed.

GOALS CONFERENCE GOALS CONFERENCE GOALS

Demonstrate and improve links between coastal research, engineering, management and decision-making.

## INTRODUCTION

The protection of coastal area and the planning of its development need a continuous and reliable assessment of the risks deriving from waves and coastal erosion, not only in the interest of public safety, but also in order to allow the recreational use of the coast and the development of the relevant infrastructure.

This is dictated not only by common sense and economical necessity but often by legal requirements; indeed, local and planning authorities often have to produce and update hazard maps and storm scenarios for the whole length of the coast in order to comply with legal requirements and economical necessities. In many Mediterranean coasts the economic interest of the beaches and the evolution of coastal erosion have put very densely populated coastal areas and infrastructure within easy reach of wave action.

1 Research Fellow, Physical Oceanography Research Laboratory, TMSI, National University of Singapore, Singapore, tmsgcc@nus.edu.sg

2 Professor of Maritime Hydraulics, Department of Civil Engineering, University of Salerno, 84084 Fisciano, Italy, epc@unisa.it

3 Research Fellow, CUGRI, 84084 Fisciano, Italy

Research on wave action on coastal structures is of course very active and plenty of results are available for the design of harbour and coastal protection structure such as groynes and breakwater; these results must be extended, adapted and verified to the special needs of risk assessment of large coastal areas. In the first place, the shape of inhabited coasts is usually rather complex, and many of the structures and buildings located near the shoreline are in a way “soft” targets, much more liable to wave damage than purpose built coastal works;  besides some infrastructures, mainly those related to tourist industry, only operate during the summer, thus requiring a specific storm return time analysis (Filianoti, 2001).

Finally, the physical dimension of the problem are often huge, as it may involve tens of kilometers of coastline, and the results have to be frequently updated to account for natural or man made changes of the areas: hence the need for efficient hazard assessment procedures, including with this term not only formulas and software, but also data collection and analysis techniques.  Analysing the risk over large coastal areas requires evaluating the run up for hundreds of similar – but not quite equal – complex geometrical configurations; the procedures have to be as objective and standardised as possible, and “first look” simplified techniques have to be provided for.  A large number of cross sections of the shore have to be surveyed in order to evaluate run up and potential hazard for coastal dwellings and infrastructure during exceptional storms.

The authors, thanks to their experience in applied research work for Local Authorities (Provincial, Town and Watershed Authorities,) have built a considerable know how on the evaluation of erosion and run up wave risk (Giarrusso et al, 1999, Boccotti et al. 2001, Cocco et al., 2001).

The final objective of these procedures is in any case the the production of hazard maps or scenarios: Fig 1, taken from Boccotti et al, shows one of such products, where a number of lines and areas are reported to mark different kinds and levels of risk.

In order to clarify the information which has to be provided when carrying out this kind of work, it is useful to introduce some definition of the terms involved.  A first distinction to be made is that between erosion and flooding hazards, the former being those deriving from the damage to houses and infrastructure caused by the removal or the collapse of the foundation ground due to beach erosion, while the flooding (or run up) hazards are those related to direct wave damage.  The two things are of course related since on the one hand it is the wave action which causes erosion, while in turn erosion processes cause changes of the shoreline that put the inhabited areas within reach of extreme wave run up.

Then a further classification should be introduced, based on the time scales involved.  Short term risk is related to the damage which might be caused by a storm, given the present configuration of the coastline; long term (n years) risk is related to the damage that a storm will cause if it occurs when the coastline is changed due to erosion within n years. Of course both risks are characterized by their probability or return period.

A coastal hazard assessment should thus provide one or more maps with different scenarios, for the short term risk and for different risk time scales n (for instance, following a common practise, n=3 and n=10 years). Each of these scenarios should report the coastline estimated position and the flooded areas for various return times and various damage level. An outline on the procedures to estimate these elements is exposed in the following.

## SURVEYING AND FORECASTING THE COASTLINE

It perhaps useful to quote from the conclusion of the PACE (Production of Aggregated Costal Evolution) project (De Vriend 1997), that “The prediction of costal behavior at space and time scales much larger than the inherent dynamic scales of the system ....is a non-trivial task”, to confirm what coastal engineers already know from practise, i.e. that while the local effects of coastal works such as harbours or groyns on the costaline can be predicted accurately enough, natural changes over great lengths are much harder to forecast.

The only way to get something more than an educated guess of what the shoreline will be in a few years, is to collect as much data as possible about the past behviour and use of them to calibrate the models which are now easily available. This is easy enough when the area is reasonably uniform in its geomorphological aspect, but may grossly fail over complex and irregular coastlines.

In any case the task surveying exceeds by far the complexity and the cost of running the models and elaborating the results so that any improvement in surveying technique is welcome.  One such improvement is certainly airborne laser topography (Breccia et al., 1999), which can provide a quick and reasonably cheap way of monitoring the coastline (Fig  2 )

Fig. 2

Conventional laser techniques, though, do not always allow a precise determination of the coastline, which can only be interpolated between measurement points on land and and at sea, thus leading possible errors, specially when the sea state is not perfectly calm. Fig 3 shows an evident misplacement of the shoreline, which is wrongly positioned inside the harbour.

Fig. 3.

Lidar survey, which can also provide depth profiles in the first few meters underwater, might provide the final answer. How often the weather and water quality conditions are sufficiently good to allow its use obviously depends on the geographical area, and there is as yet not enough experience to draw any conclusion to allow its effectiveness for the purposes of the work described in this paper.

## RETURN TIME AND DAMAGE LEVEL

We assume here that the coastline position is known, either by direct measurement (short term risk) or, as described above by extrapolating, computing or modelling the erosion processes (long term risk).

The problem is thus to determine the areas which are likely to be flooded, and which damage can be expected with different probabilities.

The hydrodynamic aspects of run up on beaches and structures have been the objects of many experiences, also within recent MAST projects, and extensive results are available both for beaches and for the shapes normally associated with dykes and breakwaters. Some of these experiences have led to simple formulas which yield the run up length, as a function of the significant wave height; other experiments yield more complex parameters such as the number of overtopping waves or the flow rate. All the formulas, however, necessarily refer to a simple geometry, such as a sea wall or a beach without obstructions.

In of real life applications however, inhabited coasts present a complex cross section because of artificial structures – such as walls, railways, buildings – vertical or nearly vertical obstacles so that run up formulas may fail. Submerged breakwater, bars and troughs may further complicate the configuration.

Recent research works (Dodd 1998, Giarrusso et al. 2000, Giarrusso et al. 2001, Hu et al. 2000, Cavallaro et al. 2001) have shown that numerical models based on Non Linear Shallow Water equations, possibly matched with Boussinesq equation to handle wave transformation (Dodd and Giarrusso 2000, Giarrusso 1998), are a useful tool for complex shapes. In particular, consistent results on typical beach-cross sections with vertical or sub-vertical obstacles can be obtained, provided that the obstacle height h is not too high compared with the wave significant height Hs, once the proper calibration is carried out. Calibration coefficients, such for instance as friction factors or velocity averaging coefficients are built in the software in various forms.

The input data for such models consists in a cross section of the coast, and a water height time history. The former can be obtained either by a topographical and bathymetric survey, or by analysing existing maps - if they exist and are reliable enough - while the latter can be simulated, once the storm parameters such as the significant wave height Hs and peak period Tp are known.    The output will then be a time history of water height and velocity in any given location along the section of the coast.

While determining the offshore storm parameters Hs and Tp as a function of their return time is a classical problem of maritime engineering, and it presents no difficulty once enough data are available, relating the time description of the run up to the expected damage is a problem in itself.  What are the tolerable limits for buildings, roads, housing, etc, in terms of extreme wave runup height, peak flow, duration, etc? There is some literature on the subject, but it is not always consistent, and it needs to be at least critically reviewed, also in connection with legal requirements.

A possible way is suggested by a table reported by Franco et al. (1994) and Mc Konnell (1998) which yields the maximum admissible average flow rate for different structures or activities, such as buildings, pedestrian walking areas, car traffic, etc.

It is thus possible do define various run up lengths by considering the distance from the still waterline where the average flow is equal to a given value, and therefore to a given damage level.  Various run up lengths can then be computed; fig.4 shows the results obtained for a simple beach profile by making use of the OTT NLSW software (Dodd et al. 1998) as a function of the significant wave height Hs. The picture also reports the run up as computed by using one of the experimental formulas (Van Der Meer 1994) available in the literature.

Fig. 4.

It is worth noticing that the run up supplied by the simple formula follows very closely the behaviour computed by the software; by properly scaling down its values, it is thus possible to use that formula (or a similar one) for drawing up hazard maps, as long as the coast profile is not too complex.

Different slopes will of course cause a change of the run up length, and may affect the reliability of simple formulas as well  (Fig  5 and  6). The reader is referred to Giarrusso et al (2000) for an outline of the effects of complex cross section and steep obstacles on the results.

Fig. 5

Fig. 6.

Once the relationship between Hs – and possibly Tp – is known, it is then an easy task to calculate the run up length, and therefore the hazard area for a given damage level. The variation of water level due to tides adds little to the conceptual difficulty and in any case it can nearly always be neglected in the Mediterranean.  (Fig  7 , 8  and  9)

Fig. 7.

Fig 8.

Fig. 9.

#### CONCLUSIONS

The problems related to coastal risk assessment are  twofold.  By making use of recent results and of advanced, but widely available technology,  wave storm hazarrd areas can be mapped with reasonable accuracy, taking into account both the return period and the expected damage level. On the other hand, the quality  of long term hazard assessing is mainly linked to the quality of erosion forecasting, which is far from being an easy task.

The best choise today is a monitoring procedure based on new techniques (Airborne Laser or Lidar) which – together with modern modelling – could supply adequate and timely information on the erosion tendency.

#### ACKNOWLEDGEMENTS

The work presented here was financed by the GNDCI (National Research Group on Hydrological and Geological Disasters) and by the CUGRI (Research Partnership on Great Hazards), through its MOVEO programme. The authors are also grateful to the Salerno Provincial Authority and to the Autorità di Bacino (Watershed Authority) Sinistra Sele for allowing the reproduction of their maps and pictures.

#### REFERENCES

Boccotti P., E. Foti, E. Pugliese Caratelli. Arena, G. Barbaro, L. Cavallaro, M.R. Della Rocca, V. Fiamma, P.Filianoti, C.C. Giarrusso, G. Spulsi 2001. Previsione e prevenzione dei danni causati dal moto ondoso sulle coste. Il Dissesto Idrogeologico: Inventario e Prospettive, XIX Giornata dell’ambiente, Accademia Nazionale dei Lincei, Rome (in italian).

Breccia S., E. Pugliese Carratelli, G. Spulsi 1999. Carte  di  Rischio  Erosione  e  Mareggiate Litorale Sinistra Sele. Convegno Asita 99,  Naples 9 – 12 November, (in italian).

Cavallaro L., C. Faraci, E. Foti, C. Giarrusso, E. Pugliese Carratelli 2001. Il run-up su spiagge e su strutture ad alta pendenza con l'algoritmo Mc Cormack-TVD. Submitted to L’acqua, (in italian).

Cocco E., C. C. Giarrusso, S. Iuliano, A. Mangiolfi, E. Pugliese Carratelli, G. Spulsi 2001. Assessment of Erosion and Wave Risk over Coastal Areas. Proceedings of Coastal Engineering 2001, 19 – 21, 19 - 21  September Rhodes, Greece.

De Vriend H.J. Prediction of Aggregated-Scale Coatal Evolution (PACE). Summary of Results project (No.: MAS3-CT95-0002): http://utsmsb002.sms.utwente.nl/ondz/pace/pace5.htm

Dodd N., Giarrusso C. C. and Nakamura S. 1998. ANEMONE: OTT-1d, A User Manual. Report TR 50, Hydraulics Research Wallingford.

Dodd N. and Giarrusso C. C. 2000. ANEMONE: OTTO-1d: A User Manual. Report TR 87, Hydraulics Research Wallingford.

Filianoti P., 2001. Return Periods of Summer-Time Sea Storms in the Central Mediterranean Sea. Proceedings of Coastal Engineering 2001, 19 – 21, September, Rhodes, Greece.

Franco L., De Gerloni M. and Van Der Meer J. W. 1994. Wave overtopping at vertical and composite breakwater. Proceedings of Coastal Engineering Conf., Kobe, Japan.

Giarrusso C. C. 1998. Studio numerico del moto ondoso su fondali intermedi e bassi. Ph.D. Thesis, Cosenza, Italy, (in italian)..

Giarrusso C. C. and Dodd N. 1997. Numerical modelling of coastal flooding. Proceedings of Waves’97 Conference, ASCE, Virginia Beach, USA.

Giarrusso C.C., Pugliese Carratelli E. and Spulsi G. 1999. Assessment Methods for Sea-Related Hazards in Coastal Areas. Journal of Natural Hazard, Kluwer Academic Publishers, pp. 295-309.

Giarrusso C.C.G, E. Foti, E. Pugliese Carratelli, 2000. Applicability of The NLSW Equations For Run-Up Evaluation Over Coasts With Quasi-Vertical Obstacles. Proceedings of Fourth International Conference on Coasts, Ports and Marine Structures, Icopmas 2000, Bandar Abbas, Iran, 21-24 November.

Giarrusso C. C., F. Dentale, E. Pugliese Carratelli 2001. Numerical Evaluation Of Crosshore Beach Sediment Transport In The Swash Zone. Proceedings of International Conference on Port And Maritime R&D And Technology, Singapore.

Hu K., Mingham C.G., Causon D.M., Ingram D.M. 1999. Numerical Modelling of Impulsive Wave Overtopping of Coastal Structure. Proceedings of Coastal Engineering and Marina Development, WIT Press, Southampton, UK.

Mc Konnell K., 1998. Revetment Systems Against Wave Attack: A design manual. Thomas Telford Publishing, London.

Van der Meer 1994. Wave run up and wave overtopping at dikes. MAST Advanced Study Course on Probabilistic Design of Reliable Coastal Structures, Bologna, Italy.